Chem 133 – Spring 2007 – Exam #1 – March 13, 2007	Name: __________________

Possibly useful equations:

V=IR			νrms=4kTRΔf		irms=2Ie−Δf

A=εbc			λ=2dnn			nλ=d(sini+sinr)

ΔrΔλ=ndcosr			vMτ=λ4		R(cm-1)=1δmax

Δδ=12Δυ

Please read the questions carefully and follow instructions.  Be sure that your handwriting is clear, and indicate your answer.  Use the correct units and a reasonable number of sig figs.  If calculations are required, show your work in order to receive full or partial credit.

1. (3 pts) Choose the one correct answer:

Which of the following is one of Kirchhoff’s Laws?

a. The current entering any point on a circuit in parallel must equal the current at all other points.

b. The voltage drop around a complete circuit in series must equal the input voltage.

c. Current is inversely proportional to resistance.

d. None of the above.

2. (4 pts) What is the voltage drop across R1 in the circuit below?

3. (5 pts)  If R1 (above) has a value of 3MΩ, what is the current (I) through the circuit?

Consider the circuit below for questions 4, 5, and 6.

15V

R1=50kΩ

R2=50kΩ

R3=30kΩ

R4=55kΩ

A

B

+

-

4. (5 pts) What is the current (I) through point B?

5. (5 pts) What is the current (I) through point A?

6. (5 pts) What is the voltage (V) (with respect to ground) at point B?

7. (4 pts) Choose the one correct answer:

As an RC circuit is charging, the following is true:

a. The voltage across the resistor is decreasing.

b. The voltage across the capacitor is increasing.

c. The current across the resistor is increasing.

d. Both a. and b. are true.

Vin

Vout

-

+

e. Both a. and c. are true.

f. Both b. and c. are true.

8. (4 pts) For an ideal op-amp the gain is ______________.

9. (6 pts) For an ideal op-amp the input resistance is infinite.  Why can’t this be true for a real op-amp (Explain in terms of Vout)?

10. (6 pts) What is the resolution (expressed in volts(V)) of an 11-bit A/D converter with an input voltage range of 0-10V?

11. (5 pts) Express the following binary numbers in their decimal form:

a. 100101	________________

b. 10		________________

12. (3 pts) Indicate if the following is true or false:

Flicker noise is generally ‘high’ frequency relative to the signal.

13. (3 pts) Indicate if the following is true or false:

Thermal noise can be completely eliminated by lowering temperature to 0oC.

14. (6 pts) Student A and Student B both carried out identical analyses on the same instrument.  Student A repeated the experiment 5 times.  Student B repeated the experiment 15 times.  By what factor is student B’s S/N better than Student A’s?

15. (5 pts) Why would someone use boxcar averaging instead of ensemble averaging?

16. (6 pts) You are designing a spectrometer.  Choose the appropriate components below to make either a (circle one): Visible spectrometer or a FTIR spectrometer.

Circle one of each of the appropriate components below for the type of spectrometer you chose above:

Light source		wavelength selector		   sample holder		   detector

a. tungsten lamp	a. monochromator		a. KBr	cell			a. photomultiplier

b. globar		b. interferometer		b. quartz cell			b. photoconductor

17. (8 pts) For a grating with a blaze spacing of 1200blazes/mm and an incident angle of 30o for incoming radiation, what is the 1st order angle of reflection (reported in degrees) for light of 400nm? (π radians = 180o; π = 3.14159).

18. (6 pts) For the same grating described in problem 17, calculate the 1st order dispersion (reported in degrees/nm); use the reflection angle that you calculated in question 17.

19. (3 pts) Choose the one correct answer:

For a Michelson interferometer, the resolution is dependent on:

a. The spacing at which measurements are made as the mirror moves.

b. The total distance that the mirror moves during a single scan.

c. The speed at which the mirror moves.

d. None of the above.

20. (8 pts) You are designing an FTIR spectrometer.  You would like your spectrometer to scan the wavenumber range 4800cm-1 to 400cm-1, and have a resolution of 0.15cm-1.  Your detector has a time constant of 5000Hz.  Specify how fast (cm/s) the interferometer mirror can move, at what mirror distance interval (cm) must signals be collected, and how far the mirror must travel in total (cm).