# HARDWARE REQUIREMENTS FOR WINDOWS NT 4.0 SERVER

- CPU sizing:

### 80486 CPU with a 33 MHz clock speed or higher

- RAM:

### 32 MB min or higher

## (16MB for operating system + 2MB for each user)

- Hard disk capacity:

### 150MB for Windows NT software

Application software

End user directories

- CD-ROM:

#### 6X CD-ROM

CD-ROM array

- Floppy disk drive

1.44/1.25 MB

- Network Interface Card (NIC):

### Auto-configuring and 32-bit with a 10BASET connector

- Bus speed:

32-bit or larger bus design

Peripheral Computer Interface (PCI)

Microchannel Architecture (MCA)

Extended Industry Standard Architecture (EISA)

- Disk controller:

Small Computer System Interface (SCSI)

Integrated Device Electronics (IDE)

Enhanced Small Device Interface (ESDI)

- Back redundancy for failure protection:

Redundant array of inexpensive drive (RAID)

- Back up system

Digital liner tape (DLT)

Digital audio tape (DAT)

Quarter-inch cartridge cassette (QIC)

Always purchase well-supported, name-brand components for the server to reduce the compatibility problems!

MINIMUM HARDWARE REQUIREMENTS FOR CLIENT WORKSTATIONS

1.	CPU sizing:

### 80486 CPU with a 33 MHz clock speed or higher

2.   RAM:

### 16 MB min or higher

3.  Hard disk capacity:

1GB or more

4.  CD-ROM:

#### 6X CD-ROM

5.  Network Interface Card (NIC):

### Auto-configuring and 32-bit with a 10BASET connector

6.  Bus speed:

32-bit or higher bus design

Peripheral Computer Interface (PCI)

Microchannel Architecture (MCA)

Extended Industry Standard Architecture (EISA)

7.  Disk controller:

Small Computer System Interface (SCSI)

Integrated Device Electronics (IDE)

Enhanced Small Device Interface (ESDI)

# Operating systems for client workstations

- NT Workstation

- Windows 95

- Windows 3.1 and 3.11

- MS-DOS

# WINDOWS NT 4.0 SERVER INSTALLATION

- Turn of the computer

- Place Windows NT Server Setup Disk #1 in drive A

- Turn on the computer, which boots from Setup Disk #1

- Follow on screen request

- Insert Windows NT Server Setup Disk #2 into drive A

- Press [Enter]

- Press [Enter] to select “To set up Windows NT now” option

- Press [Enter] to select “To continue” option

- Follow on screen request

a)   Insert Windows NT Server Setup Disk #3 into drive A

# b)   Press [Enter]

- Press [Enter] to continue the installation

- Follow on screen request

- Insert the Windows NT Server CD-ROM into CD-ROM drive

- Press [Enter]

- Use Page Up and Page Down keys to read the Microsoft license agreement and press F8 to agree the terms of the license

- Highlight the line “No Changes: the above list matches my computer” and press [Enter]

- Highlight the line “To install Windows NT on the highlighted partition or unpartitioned space, Press ENTER” and press [Enter]

- Highlight the line “Convert the partition to NTFS” and press [Enter]

- Highlight the line “To format the drive, press F” and press F

- Accept the default option “\WINNT” as the directory path and name for the NT server and press [Enter]

- Highlight the line “ To allow Setup to perform an exhaustive secondary examination of your hard disk(s), press ENTER” and press [Enter]

- Remove all media from the floppy and CD-ROM drives and press [Enter] to reboot the computer

- Insert the Windows NT Server CD-ROM into CD-ROM drive and click OK option

- Select the line “1) Gathering information about your computer” and click Next option

- Enter your group name for Name and MIS for Organization

- Click Next option

- Enter 10-digit CD Key (on the back of the Windows NT Server CD-ROM) and click Next option

- Select the line “Per Server concurrent connections” and enter 20 in the box

- Click Next option

- Enter SANDMAN for the name of the server and click Next option

- Click the radio button in the front of the line “Primary Domain Controller”

- Click Next option

- Enter your group name for Password and Confirm Password

- Click Next option

- Click the radio button in the front of the line “Yes, create an emergency repair disk (recommended)” and click Next option

- Click the Next option to accept the default software for the Select Components screen

- Select the line “2) Installing Windows NT Networking” and click Next option

- Click the box in the front of the line “Wired to the network” and click the Next option

- Leave the box blank in the front of the line “ Install Microsoft Internet Information Server” and click Next option

- Click the “Start Search” button to search the network interface card

- Click Next option after the Setup display the name of the network interface card

- Click “NetbEUI Protocol” from the box of Network Protocols

- Click Next option

- Select “NETBIOS Interface”, “Workstation”, and “Server” from the box of Network Services and click Next option

- Click Next option to install selected components

- Click Continue option to accept the Setup’s NIC configuration settings detection

- Click Next option to accept the bindings as created by Setup for better performance

- Click Next option to start the network

- Enter MISTELCOM for the domain and click Next option

- Select the line “3)Finishing Setup” and click Finish

- Click Date & Time tab from Date/Time Properties Screen and enter the current date and the current time

- Click Time Zone tab and use the down arrow of the scroll box to select the proper time zone

- Be sure that there is a check mark in the box in the front of the line “Automatically adjust clock for daylight saving changes”

- Click OK option

- Click the Test option to check that the color and resolution are accurate

- Click OK option to proceed

- Insert a blank disk into drive A to create an Emergency Repair Disk

- Remove all disks from floppy and CD-ROM drives and click Restart Computer button

- Press [Ctrl]+{Alt]+[Del] at the same time to reboot the NT Server

- Log on using Administrator account and your group name as password

- Click Start button and click Shot Down option

- Wait for the permission message from NT Server and turn off the computer

NT SERVER MANAGEMENT

- Turn on the power for NT Server (CPU and Monitor)

- Log on using Administrator account and your group name as password

Setting Account Policies

1.   Click Start button and highlight the Program option

2.   Highlight the Administrative Tools option and click User Manager for Domain

- Click Policies menu on the top of the screen and click Account option

- Enter the following for the Account Policy Screen

- Click radio button for Expires in __Days and enter 60 in the text box

- Click the radio button for Allow Changes Immediately

- Click radio button for At Least __ Characters and enter 6 in the text box

- Click radio button for Remember  __ Passwords and enter 5 in the text box

- Click radio button for Account lockout

- Enter 5 in the text box of Lockout after __ bad lock attempts

- Enter 30 in the text box of Reset count after __ minutes

- Click radio button for Duration __ minutes and enter 30 in the text box

- Place a check in the box before the line “User must log on in order to change password

- Click OK option to save the settings and changes

# Creating Accounts

1.    Click User menu on the top of the screen and click New User

2.    Enter your initial and last name (without a space) for Username

- Enter your full name (without space) for Full Name

- Enter Network Administrator for Description

- Enter your full name (without space) for Password and Confirm Password

- Do not place any check in the boxes before the lines “User Must Change Password at Next Logon’, “ User Cannot Change Password”, “Password Never Expires”, and “Account Disabled”

- Click Group button

- Highlight Administrator and click Add button

- Click OK option to save the settings or changes

- Click Profile button

- Click the radio button for Connect __ to __ and enter H: for the first text box and \USERS\YOUR LAST NAME for the second text box

- Click Hours button

- Click the intersection box of Saturday and Midnight and click disable

- Click OK option to save the setting or change

- Click Hours button

- Click Cancel option to save the setting or change

- Click Logon button

- Click the radio button before the line “User May Log on to All Workstation

- Click OK option to save the change

- Click Account button

- Click the radio button before “Never”

- Click the radio button before the line “Global Account”

- Click OK option to save the settings or changes

- Click Dialin button

- Click the radio button before the line “Grant dialin permission to user”

- Click the radio button before the line “No Call Back”

- Click OK option to save the settings or changes

- Click Add option to add a new user

# Creating Groups

- Click User menu on the top of the screen and click New Global Group

- Enter Managers for Group Name

- Enter MIS Managers for Description

- Highlight a name of your group to be included as a member from the right box and click add (you may need to repeat this step to include all your group members in the group)

- Click OK option to save the changes

# Creating Rights

- Click Policies menu on the top of the screen and click User Rights

- Highlight the line “Access This Computer From Network” and click Add button

- Highlight the line “Managers      MIS Managers” and click add button

- Click OK button

- Click OK button to save the changes

6.   Click User menu on the top of the screen and click exit

7.   Click Start button and click Shot Down option

- 8.   Wait for the permission message from NT Server and turn off the computer

Creating User Subdirectory

- 1.   Turn on the power for NT Server (CPU and Monitor)

- Log on using Administrator account and your group name as password

- Double click My Computer icon on the desktop

- Double click hard drive C

- Double click Users folder

- Click File menu, New, and Folder

- Type YOUR LAST NAME in the box underneath the folder and press Enter

- Click Start button and click Shot Down option

- 9.    Wait for the permission message from NT Server and turn off the computer

*****Create user subdirectory before creating user account*****

PREPARING WINDOWS 95 OF WORKSTATION FOR NETWORK

- Turn on power for workstation

- Click Start, Settings, and Control Panel

- Double click the Network icon

- Click Add button

- Highlight the Client option and click Add button

- Select the Microsoft in the Manufacturers list box

- Select the Client for Microsoft Networks

- Click OK

- Click Start, Settings, and Control Panel

- Highlight the Client for Microsoft Networks option

- Click the Properties

- Place a check in the box for Log on to Windows NT domain

- Type MISTELCOM for domain name

- Click the radio button for Log on and restore network connection

- Click OK

- Click the File and Print Sharing button on the Network Configuration screen

- Place a check in the box to share files

- Click the Identification tab on the Network screen

- Type your group name for the computer name

- Type MISTELCOM for the workgroup name

- Type PRACTICE for computer description

- Click OK

- Click the Configuration tab on the Network screen

- Click Add button

- Click Protocol

- Highlight Microsoft under Manufacturers list box

- Select TCP/IP in the Network Protocols list box

- Click OK

- Click Start, Shut Down, Shut down computer?, and Yes

- Wait for the permission to turn off the computer power