EEE 186    	COMMUNICATIONS SYSTEMS LABORATORY         	FALL  2020

Instructor	:   	Dr. Preetham B. Kumar

Office hours:  	:	Tues/Wed/Thurs: 2 - 3 pm or by appointment

https://csus.zoom.us/j/867091331

Class hours       :	Thursday  10.30 am – 1 p.m

Zoom link:  https://csus.zoom.us/j/94007224714

Telephone	:   	916-278-7949

E-mail		:   	kumarp@ecs.csus.edu

Website	:	https://www.csus.edu/faculty/k/preetham.kumar/

Prescribed Text : 	Communications System Laboratory by B.P. Kumar,

CRC Press, 2015.

ISBN No.: 978-1-4822-4544-8

References:	MATLAB/SIMULINK Guide, The Mathworks.

http://www.mathworks.com/

Course Grading

Laboratory  1:	15%

Laboratory  2:	15%

Laboratory  3:	15%

Laboratory  4:	15%

Laboratory  5:	15%

Laboratory  6:	15%

Attendance: 10%

Attendance

Attendance in the class is required to receive credit for hardware section of lab reports. However, software lab work may be completed after lab hours, if required.

Fall 2020 Lab schedule

WEEK	        LAB DATE			TOPIC/SECTION FROM BOOK

_____________________________________________________________________________

1		09/03/20		Lab 1:  Introduction to software and equipment/

C1.1 – C1.5 - Software

2		09/10/20		H1.1 – H1.3 - Hardware

3		09/17/20		Lab 2: Amplitude Modulation (AM)

C3.1 – Simulation

4		09/24/20		H3.1 – Fabrication and testing

5		10/01/20		Lab 3: Frequency Modulation (FM)

C3.2 – Simulation

6		10/08/20		H3.2 – Fabrication and testing

7		10/15/20		Lab 4: Binary Phase Shift Keying (BPSK)

C4.4 – Simulation

8		10/22/20		H4.2 – Fabrication and testing

9		10/29/20		Lab 5: Frequency Shift Keying (FSK)

C4.6 – Simulation

10		11/05/20		H4.3 – Fabrication and testing

11		11/12/20		Lab 6: Direct Sequence Spread Spectrum (DSSS)

12		11/19/20		C5.2 – Simulation

13		11/26/20		Finish up Labs

14		12/03/20		Finish up Labs

15		12/10/20		Finish up Labs

16		12/17/20		Finals Week-No Lab (May use for make up labs)

EEE 186		MAT LAB COMMANDS AND TOOLBOXES

System operating commands

PC based MATLAB can be opened from either by clicking on the MATLAB icon or by entering 'mat lab' at the DOS prompt, and return.  The MATLAB prompt is >>, which indicates that commands can be started, either line by line, or by running a stored program.  A complete program, consisting of a set of commands, can be stored in a MATLAB file for repeated use as follows:

(a)  Open a file in any text editor ( either in MATLAB or otherwise), and write the program.

(b)  After writing the program, exit saving  as a filename's file.

(c)  To run the program, type the filename after the prompt:

>> filename

The program will  run, and the results and error messages, if any,  will be displayed on the screen. Plots will  appear on a new screen.

I.  NUMBERS

Generation of  numbers

Example: Generate the real numbers z1 = 3, z2 = 4.

>>  z1 = 3

>>  z2 = 4

Example: Generate the complex numbers z1 = 3+j4, z2 = 4+j 5

>>  z1 = 3+j*4

>>  z2 = 4+j*5

Note:  The symbol I can be used instead of j to represent  v-1.

Example:  Find the magnitude and phase of the complex number 3+j*4

>>  z = 3+j*4

>>  zm = abs(z)			; gives the magnitude of z

>>  zp = angle(z)			; gives the phase of z in radians

Addition or Subtraction of Numbers  (real or complex)

>>  z = z1 + z2				; addition

>>  z = z1 - z2				; subtraction

Multiplication or Division of Numbers (real or complex)

>>  z = z1*z2				; multiplication

>>  z = z1/z2				; division

II.  VECTORS

Generation of  vectors

Example: Generate the vectors x = [1 3 5] and y = [ 2 0 4 5 6]

>>  x = [1 3 5]			 ; generates the vector of length 3

>>  y = [2 0 4 5 6]			; generates the vector of length 5

Addition or Subtraction of Vectors x and y  of same length

>>  z = x+ y				; addition

>>  z = x - y				; subtraction

Multiplication or Division of Vectors x and y  of same length

>>  z = x. * y				; multiplication

>>  z = x. / y				; division

Note: The dot after x is necessary since x is a vector and not a number.

## MATLAB TOOLBOXES

## MATLAB commands are divided into different toolboxes depending on the applications. Various toolboxes developed by MATLAB include:

Communication Toolbox

Image Processing Toolbox

## Signal Processing Toolbox

## Fuzzy Logic Toolbox

## Spline Toolbox

## NAG Foundation Toolbox

## Neural Network Toolbox

## Nonlinear Control Design Toolbox

## Statistics Toolbox

## Optimization Toolbox

## Symbolic Math Toolbox

## Partial Differential Equation Toolbox

## System Identification Toolbox

## PROGRAMMING WITH VECTORS

Programs involving vectors can be written using either FOR LOOPS or VECTOR commands. Since MATLAB is basically a vector based program, it is often more efficient to write programs using VECTOR commands. However, FOR LOOPS give a clearer understanding of the program, especially for the beginner:

Example:  Sum the following series:

S = 1 + 3 + 5  . . . . . . .99.

### FOR LOOP approach

>>  S = 0.0				; initializes the sum to zero

>>  for  i = 1 : 2 : 99

S = S + i

end

>> S					; gives the value of the sum

### VECTOR approach

>> i =1 : 2 : 99;			; creates the vector i

>> S = sum ( i );			; obtains the sum S

Example:   Generate the discrete-time signal y(n) = n sin(n/2) in the interval 0  n  10.

### FOR LOOP approach

>> for  n = 1:1: 11

n1 = n - 1

y(n) = n1 * sin(pi*n1/2)

end

>>  y					; gives the vector y

>> n = 0:1:10				; generates the vector n

>> stem(n,y)				; plots the signal y vs. n with impulses

### VECTOR approach

>> n = 0 : 10;				; creates the vector n

>> y = n.*sin(pi*n/2);			; obtains the vector y

>> stem(n,y)				; plots the signal y vs. n with impulses

SIMULINK COMMANDS AND EXAMPLES

After logging into MATLAB, you will receive the prompt  >>. In order to open up SIMULINK, type in the following:

>> simulink

GENERAL SIMULINK OPERATIONS

Two windows will open up:  the model window and the library window.  The model window is the space utilized for creating your simulation model.   In order to create the model of the system, components will have to be taken from the library using the computer mouse, and inserted into the model window.

If you browse the library window, the following sections will be seen. Each section can be accessed by clicking on it.

- Sources - This section consists of different signal sources such as sinusoidal, triangular, pulse, random or files containing audio or video signals.

- Sinks  -  This section consists of measuring instruments such as scopes and displays

- Linear - This section consists linear components performing operations like

summing, integration, product.

- Nonlinear - Nonlinear operations

- Connections - Multiplexers, Demultiplexers

- Blocksets and Toolboxes - These specify different areas of SIMULINK

- Communications

- DSP

- Neural Nets

- Simulation Extras

EDITING, RUNNING AND SAVING SIMULINK FILES

The complete system is created in the model window by utilizing components from the various available libraries.  Once a complete model is created, save the model  into a file.  Click on Simulation and select Run.  The simulation will run, and the output plots can be displayed by clicking on the appropriate sinks.  Save the output plots also into files.  The model and output files can be printed out from the files.

DEMO FILES

Try out the demo files, both in the main library window, and in the Toolboxes window.  There are several illustrative demonstration files in the areas if signal processing, image processing and communications.

Some examples are given below:

Continuous-time system

x(t) = A sin(t)

n(t)

Analog signal

Pseudo-random noise

Time scope

+

Time scope

Time scope

(a) Run the simulation for sinusoidal signal, x(t),  amplitude of 5 Volts and frequency  = 10 rad./s. The signal n(t) is a pseudo-random noise with maximum amplitude of 0.5 volts.

Observe the combined signal on the time scope, and familiarize yourself with the settings.

- (b)  Try changing the sinusoidal signal amplitude (2V, 10V),  and frequency (20 rad./s, 50 rad./s), and observe the output on the time scope.

- Discrete-time system

+

0.4

z-1

x(n)

y(n)

z/(z2+z-0.3)

- (a) Observe the output signal on the time scope, for an input periodic pulse generator having the following parameters: Pulse amplitude 1 V, Pulse period 2 seconds and  pulse width of 1 second.

(b) Try changing the input signal amplitude (2 V, 3V) and  pulse width (0.5, 1.5 sec.), and observe on the time scope.