# Chapter 6 Solutions

1.	OT = 450 minutes

a.	Minimum cycle time = length of longest task, which is 2.4 minutes.

Maximum cycle time =  task times = 18 minutes.

b.	Range of output:

@2.4min.:4502.4=187.5units@18min.:45018=25units

c.	N=DxtOT=187.5(18)450=7.5, which rounds to 8

d.	Output=OTCT  Solving for CT, CT=450125=3.6 minutes per cycle

e.	Potential output:

(1) CT=9min.:OTCT=4509=50units

(2) CT=15min.:45015=30units

0.6

0.7

0.8

0.5

1.0

0.5

0.5

1.4

c

f

h

g

d

e

b

a

2.

Desired output = 33.33 units per hour

Operating time = 60 minutes per hour

CT=Operating timeDesired output=60 minutes per hour33.33 units per hour=1.80 minutes per unit

Solutions (continued)

a.

| Task | Number of following tasks | Positional Weight |
| --- | --- | --- |
| A | 7 | 6 |
| B | 6 | 4.6 |
| C | 2 | 1.6 |
| D | 2 | 2.2 |
| E | 2 | 2.3 |
| F | 1 | 1.0 |
| G | 1 | 1.5 |
| H | 0 | 0.5 |

Assembly Line Balancing Table  (CT = 1.8)

| Work Station | Task | Task Time | Time Remaining | Feasible tasks Remaining |
| --- | --- | --- | --- | --- |
| I | A | 1.4 | 0.4 | – |
| II | B | 0.5 | 1.3 | C, D, E |
|  | E | 0.8 | 0.5 | – |
| III | D | 0.7 | 1.1 | C |
|  | C | 0.6 | 0.5 | F |
|  | F | 0.5 | 0 | – |
| ### IV | G | 1.0 | 0.8 | H |
|  | H | 0.5 | 0.3 | – |

b. 		Assembly Line Balancing Table  (CT = 1.8)

| Work Station | Task | Task Time | Time Remaining | Feasible tasks Remaining |
| --- | --- | --- | --- | --- |
| I | A | 1.4 | 0.4 | – |
| II | B | 0.5 | 1.3 | C, D, E |
|  | E | 0.8 | 0.5 | – |
| III | D | 0.7 | 1.1 | C |
|  | C | 0.6 | 0.5 | F |
|  | F | 0.5 | 0 | – |
| ### IV | G | 1.0 | 0.8 | H |
|  | H | 0.5 | 0.3 | – |

Solutions (continued)

c.	Efficiency=Total timeCTx no. of stations=6.07.2=83.3%

3

a

d

f

7

5

2

b

4

c

4

e

9

h

5

i

6

g

3.

Desired output = 4

Operating time = 56 minutes

CT=Operating timeDesired output=56 minutes per hour4 units per hour=14 minutes per unit

| Task | # of Following tasks | # Positional Weight |
| --- | --- | --- |
| A | 4 | 23 |
| B | 3 | 20 |
| C | 2 | 18 |
| D | 3 | 25 |
| E | 2 | 18 |
| F | 4 | 29 |
| G | 3 | 24 |
| H | 1 | 14 |
| I | 0 | 5 |

Solutions (continued)

a.	First rule: most followers. Second rule: largest positional weight.

Assembly Line Balancing Table  (CT = 14)

| Work Station | Task | Task Time | Time Remaining | Feasible tasks Remaining |
| --- | --- | --- | --- | --- |
| I | F | 5 | 9 | A,D,G |
|  | A | 3 | 6 | B,G |
|  | G | 6 | – | – |
| II | D | 7 | 7 | B, E |
|  | B | 2 | 5 | C |
|  | C | 4 | 1 | – |
| III | E | 4 | 10 | H |
|  | H | 9 | 1 | – |
| ### IV | I | 5 | 9 | – |

b. 	First rule: Largest positional weight.

## Assembly Line Balancing Table  (CT = 14)

| Work Station | Task | Task Time | Time Remaining | Feasible tasks Remaining |
| --- | --- | --- | --- | --- |
| I | F | 5 | 9 | A,D,G |
|  | D | 7 | 2 | – |
| II | G | 6 | 8 | A, E |
|  | A | 3 | 5 | B,E |
|  | B | 2 | 3 | – |
| III | C | 4 | 10 | E |
|  | E | 4 | 6 | – |
| ### IV | H | 9 | 5 | I |
|  | I | 5 | – |  |

c.	Efficiency=Total timeCTx no. of stations=4556=80.36%

Solutions (continued)

a

b

d

c

f

e

g

h

4.	a, b

a.	l.

2. 	Minimum Ct = 1.3 minutes

|  | Task | Following tasks |
| --- | --- | --- |
|  | a | 4 |
|  | b | 3 |
|  | c | 3 |
|  | d | 2 |
|  | e | 3 |
|  | f | 2 |
|  | g | 1 |
|  | h | 0 |

| Work Station | Eligible | Assign | Time Remaining | Idle Time |
| --- | --- | --- | --- | --- |
| I | a | A | 1.1 |  |
|  | b,c,e, (tie) | B | 0.7 |  |
|  |  | C | 0.4 |  |
|  |  | E | 0.3 | 0.3 |
| II | d | D | 0.0 | 0.0 |
| III | f,g | F | 0.5 |  |
|  |  | G | 0.2 | 0.2 |
| ### IV | h | H | 0.1 | 0.1 |
|  |  |  |  | 0.6 |

3.	Idle percent=(idle time)NxCT=.64(1.3)=11.54percent

4.	Output=OTCT=420 min./day1.3min.cycle=323.1(roundsto323)unitsday

Solutions (continued)

b.	1.	Total time=4.6,CT=TotaltimeN=4.62=2.3minutes

2. 	Assign a, b, c, d, and e to station 1: 2.3 minutes [no idle time]

Assign f, g, and h to station 2: 2.3 minutes

3. 	Output=OTCT=420 2.3=182.6unitsday

4. 	Maximum Ct is 4.6. Output=420 min./day4.6 min.cycle=91.30unitsday

a

b

d

c

f

e

g

.2

.4

.2

1.2

1.0

1.2

.4

5.	a.

b. 	The minimum cycle time = maximum task time =1.2 minutes

The maximum cycle time = .2 +.4 +.2 +.4 +1.2 +1.2 + 1.0 = 4.6 minutes

CT=OToutput=480 min./day240 unitsday=2 minutes (calculated CT)

c.	N=tCT=4.62.0=2.3(rounds to 3) stations

d.

| Task | Number of following tasks |
| --- | --- |
| A | 4 |
| B | 3 |
| C | 2 |
| D | 2 |
| E | 1 |
| F | 1 |
| G | 0 |

Solutions (continued)

Assembly Line Balancing Table  (CT = 2 minutes)

| Work Station | Task | Task Time | Time Remaining | Feasible tasks Remaining |
| --- | --- | --- | --- | --- |
| I | A | 0.2 | 1.8 | B,D |
|  | B | 0.4 | 1.4 | C, D |
|  | D | 0.4 | 1.0 | C |
|  | C | 0.2 | 0.8 | – |
| II | E | 1.2 | 0.8 | – |
| ### III | F | 1.2 | 0.8 | – |
| ### IV | G | 1.0 | 1.0 | – |

e.	Idle percent=.8+.8+.8+1.0(4)(2)=3.48.0=42.5%

E=4.6(4)(2)=57.5%

## 6.	a.

0.1

(3.9)

0.2

(3.8)

0.9

(3.6)

0.6

(2.7)

0.1

(2.2)

0.2

(2.1)

0.4

(1.9)

0.1

(1.5)

0.2

(1.4)

0.7

(1.2)

a

b

c

d

e

f

g

h

i

j

k

l

(0.5)

(0.2)

0.3

0.2

(1,2)

CT = 1.5

Positional weights

in parentheses

Solutions (continued)

- b. 	Using both the greatest positional weight and the greatest number of following tasks rules result in the following balance.

Assembly Line Balancing Table  (CT = 1.5 minutes)

| Work Station | Task | Task Time | Time Remaining | Feasible tasks Remaining |
| --- | --- | --- | --- | --- |
| I | A | 0.1 | 1.4 | B |
|  | B | 0.2 | 1.2 | C |
|  | C | 0.9 | 0.3 | E |
|  | E | 0.1 | 0.2 | – |
| II | D | 0.6 | 0.9 | F |
|  | F | 0.2 | 0.7 | G |
|  | G | 0.4 | 0.3 | H |
|  | H | 0.1 | 0.2 | I |
|  | I | 0.2 | – | – |
| III | J | 0.7 | 0.8 | K |
|  | K | 0.3 | 0.5 | L |
|  | L | 0.2 | 0.3 | – |

## Total idle time = 0.2 + 0 + 0.3 = 0.5

c.	For positional weights and greatest number of following tasks

Idle rate=0.5x1003(1.5)=11.11%

a

b

c

e

d

h

g

f

i

j

7.	a.

Solutions (continued)

b.	CT=OTD=7(60)500=.84 minutes = 50.4 seconds (maximum cycle time)

Minimum cycle time = maximum task time = 45 seconds  (results in 560 units of production)

c.	N=tCT=19350.4=3.83 or 4 stations

## d.

| Task | Number of followers | *PW |
| --- | --- | --- |
| A | 6 | 106 |
| B | 5 | 61 |
| C | 4 | 50 |
| D | 4 | 106 |
| E | 3 | 56 |
| F | 2 | 30 |
| G | 2 | 31 |
| H | 2 | 29 |
| I | 1 | 19 |
| J | 0 | 10 |

*Positional weight

CT = 50 seconds

| Work Station | Task | Task Time | Time Remaining | Feasible tasks Remaining |
| --- | --- | --- | --- | --- |
| I | A | 45 | 5 | – |
| III | D | 50 | – | – |
| III | B | 11 | 39 | C, E |
|  | E | 26 | 13 | C, F |
|  | C | 9 | 4 | – |
| IV | G | 12 | 38 | H, F |
|  | F | 11 | 27 | H |
|  | H | 10 | 17 | I |
|  | I | 9 | 8 | – |
| V | J | 10 | 40 | – |

Solutions (continued)

e.	I=1−193(50)(5)=22.8%

a

c

e

j

k

m

g

h

d

f

i

b

8.

## b.	CT=200400=2.0 minutes

|  | Station | Tasks | Time | Idle/Time |
| --- | --- | --- | --- | --- |
|  | 1 | a,b | 1.9 | .1 |
|  | 2 | c,d | 1.9 | .1 |
|  | 3 | e,f,i | 2.0 | 0 |
|  | 4 | g,h,j | 1.5 | .5 |
|  | 5 | k,m | 1.2 | .8 |
|  |  |  |  | 1.5 |

|  | c. | Tasks | Positional Weight |
| --- | --- | --- | --- |
|  |  | a | 8.5 |
|  |  | b | 4.6 |
|  |  | c | 4.4 |
|  |  | d | 4.2 |
|  |  | e | 3.2 |
|  |  | f | 3.5 |
|  |  | g | 1.9 |
|  |  | h | 1.5 |
|  |  | i | 2.5 |
|  |  | j | 2.0 |
|  |  | k | 1.2 |
|  |  | m | .3 |

Solutions (continued)

| Work Station | Task | Task Time | Time Remaining | Feasible tasks Remaining |
| --- | --- | --- | --- | --- |
| I | a | .5 | 1.5 | b, c, d |
|  | b | 1.4 | .1 | – |
| II | c | 1.2 | .8 | d, e |
|  | d | .7 | .1 |  |
| III | f | 1.0 | 1.0 | e, i |
|  | e | .5 | .5 | i, g |
|  | i | .5 | 0 | – |
| IV | j | .8 | 1.2 | g |
|  | g | .4 | .8 | h |
|  | h | .3 | .5 | – |
|  | k | .9 | 1.1 | m |
|  | m | .3 | .8 | – |

Total idle time = .1 + .1 + 0 + .5 + .8 = 1.5 minutes

d.	Balance delay: part b and c 1.5/10 = 15%

| 10. 1 2 3 4 5 6 A o o o A X o o o o A o o o o 9. | 1 | 4 | 3 |
| --- | --- | --- | --- |
|  | 2 | 5 | 6 |
| 11. | 1 | 5 | 4 |
|  | 3 | 8 | 7 |
|  | 6 | 2 |  |

## Solutions (continued)

| 12. | 3 | 1 | 4 | 8 | Or | 5 | 1 | 4 | 8 | Or | 5 | 1 | 4 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 2 | 7 | 6 |  | 3 | 2 | 7 | 6 |  | 3 | 2 | 7 | 8 |
|  | 5 | 1 | 7 | 8 | Or | 3 | 1 | 7 | 8 | Or | 3 | 1 | 4 | 6 |
|  | 3 | 2 | 4 | 6 |  | 5 | 2 | 4 | 6 |  | 5 | 2 | 7 | 8 |
|  | 3 | 1 | 7 | 6 | Or | 5 | 1 | 7 | 6 |  |  |  |  |  |
|  | 5 | 2 | 4 | 8 |  | 3 | 2 | 4 | 8 |  |  |  |  |  |

| 13. | 3 | 1 | 8 |
| --- | --- | --- | --- |
|  | 9 | 7 | 4 |
|  | 5 | 2 | 6 |

- First rank or arrange the number of trips from high to low.

- Department	Number of trips

  - 90

  - 80

  - 55

  - 40

- From this we can see that departments 2 and 4 have the greatest interdepartmental work flow, so they should be close, perhaps locations C and B. Next, we can see that the work flows for 1 and 4, and 3 and 4 are high. Therefore department 4 has to be located at a central location (location B), while department 2 is in location C, department 1 is in location A, and department 3 is in location D.

Distance * Number of trips matrix

| Department | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| 1 | – | (10 x 80) = 800 | (20x 70) = 1400 | (80 x 40) = 3200 |
| 2 | – | – | (40 x 60) = 2400 | (90 x 40) = 3600 |
| 3 | – | – | – | (55 x 50) = 2750 |
| 4 | – | – | – | – |

Total cost = $14,150

Solutions (continued)

| 15. | No. of trips (two way) |  |  | Order of Assignment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1–2 | 10 |  |  |  |  |  |  |  |  |
|  | 1–3 | 5 |  |  | A reasonable (intuitive) set of assignments is: |  |  |  |  |  |
|  | 1–4 | 90 |  | 11 |  |  |  |  |  |  |
|  | 1–5 | 370 |  | 1 |  | A #1 | B #5 |  |  |  |
|  | 1–6 | 135 |  | 6 |  |  |  |  |  |  |
|  | 1–7 | 125 |  | 7 |  | C #7 | D #4 | E #3 |  |  |
|  | 1–8 | 0 |  |  |  |  |  |  |  |  |
|  | 2–3 | 360 |  | 2 |  | F #6 | G #2 | H #8 |  |  |
|  | 2–4 | 120 |  | 8 (tie) |  |  |  |  |  |  |
|  | 2–5 | 40 |  |  |  |  |  |  |  |  |
|  | 2–6 | 115 |  | 9 | This set of assignments has a total cost of $143,650 per day. |  |  |  |  |  |
|  | 2–7 | 45 |  |  |  |  |  |  |  |  |
|  | 2–8 | 120 |  | 8 (tie) |  |  |  |  |  |  |
|  | 3–4 | 350 |  | 3 | Slight variations would also be reasonable, as long as departments 2, 4 and 8 are close to 3, 4 is close to 5, and 5 is close to 1. |  |  |  |  |  |
|  | 3–5 | 110 |  | 10 |  |  |  |  |  |  |
|  | 3–6 | 40 |  |  |  |  |  |  |  |  |
|  | 3–7 | 20 |  |  |  |  |  |  |  |  |
|  | 3–8 | 200 |  | 4 |  |  |  |  |  |  |
|  | No. of trips (two way) |  |  | Order of Assignment |  |  |  |  |  |  |
|  | 4–5 | 190 |  | 5 (tie) |  |  |  |  |  |  |
|  | 4–6 | 70 |  | 12 |  |  |  |  |  |  |
|  | 4–7 | 50 |  |  |  |  |  |  |  |  |
|  | 4–8 | 190 |  | 5 (tie) |  |  |  |  |  |  |
|  | 5–6 | 10 |  |  |  |  |  |  |  |  |
|  | 5–7 | 40 |  |  |  |  |  |  |  |  |
|  | 5–8 | 10 |  |  |  |  |  |  |  |  |
|  | 6–7 | 50 |  |  |  |  |  |  |  |  |
|  | 6–8 | 20 |  |  |  |  |  |  |  |  |
|  | 7–8 | 20 |  |  |  |  |  |  |  |  |

## Solutions (continued)

| 16. | No. of trips (two way) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1–2 | 0 |  |  |  |
|  | 1–3 | 40 |  |  | A reasonable (intuitive) assignment is: |
|  | 1–4 | 110 |  |  | 3–A, 5–B, 1–C, 4–D, 6–E, 2–F. |
|  | 1–5 | 80 |  |  | An equivalent solution is the reverse order: |
|  | 1–6 | 50 |  |  | 2–F, 6–B, 4–C, 1–D, 5–E, 3–F. |
|  | 2–3 | 0 |  |  |  |
|  | 2–4 | 50 |  |  |  |
|  | 2–5 | 40 |  |  |  |
|  | 2–6 | 120 |  |  |  |
|  | 3–4 | 10 |  |  |  |
|  | 3–5 | 250 |  |  |  |
|  | 3–6 | 10 |  |  |  |
|  | 4–5 | 40 |  |  |  |
|  | 4–6 | 90 |  |  |  |
|  | 5–6 | 20 |  |  |  |
|  | (Ignore Reception since all locations are the same distance from it.) |  |  |  |  |

- 17.	Two-way trips can not be used here because of the one-way route restriction. Consequently, students are forced to develop a heuristic that will yield reasonable assignments. One possible heuristic is the following:

- Beginning with Department 1, identify the department which receives the greatest number of trips from that department (e.g., 40 to Department 2). Assign that department to the next location counter-clockwise.

- For that department (e.g., 2) identify the department which receives the greatest number of trips (e.g., 5) and assign it to the next position.

- Continue in this manner until all departments have been assigned.

- The resulting set of assignments for this problem is: A–1, 2–B, 5–C, 4–D, 9–E, 8–F, 6–G, 10–H, 7–I, 3–J.

- Students may raise the question about return trips to the original departments after delivery, which would seem to make all locations comparable. Three possible explanations are:

- 1.	Return trips cost less because they are unloaded.

- 2.	Unloaded trips may be permitted to move clockwise.

- 3.	Material handlers (?) pick up new load at each new department and move it to the next department.

(The last explanation seems to appeal most to students.)

## Solutions (continued)

| Work Station | Task | Task Time | Time Remaining | Feasible tasks Remaining |
| --- | --- | --- | --- | --- |
| I | D | 50 | 0.4 | – |
| II | A | 45 | 5.4 | – |
| III | E | 26 | 24 | B ,F |
|  | B | 11 | 13 | C, F |
|  | F | 11 | 2.4 | – |
| IV | C | 9 | 41 | G, H |
|  | G | 12 | 29 | H |
|  | H | 10 | 19 | I |
|  | I | 9 | 10 | J |
|  | J | 10 | 0.4 |  |